# Chapter 5: System Development

## 5.1 Navigation/Module Structure

The system is designed as a distributed IoT architecture comprising three main modules: the Gate Unit (Raspberry Pi), the Bus Unit (ESP32), and the Cloud Backend (Google Workspace). This modular approach ensures that attendance tracking covers both the school premises and transportation.

### 1. Gate Unit Module (Raspberry Pi)

This module acts as the primary entry/exit checkpoint. It is a "Thick Client" that stores a local copy of the student database to ensure functionality even during internet outages.

Input Sub-module: Handles RFID card reading via SPI interface.

Logic Sub-module: Validates the scanned ID against the local JSON database, checks for anti-passback (cool-down timer), and determines if the event is an Entry or Exit.

Communication Sub-module: Sends SMS notifications directly via GSM and syncs logs to the cloud in the background.

### 2. Bus Unit Module (ESP32)

Designed for mobility, this module is a "Thin Client" that offloads validation logic to the cloud to minimize hardware complexity.

Network Handler: Manages GPRS connectivity using the SIM800L module.

Hardware Interface: Controls the RFID reader and provides audio-visual feedback (LED/Buzzer).

Cloud Connector: Sends secure HTTPS requests to the Google Apps Script API for validation.

### 3. Cloud Module (Google Script & Sheets)

Serves as the central database and API.

Database: A Google Sheet acts as the relational database, storing Student Profiles and Attendance Logs.

API Layer: Google Apps Script exposes endpoints (doGet, doPost) to receive data from the IoT devices and return validation results.

## 5.2 Development Environment

### Hardware Environment

Raspberry Pi 3/4: Serving as the core processor for the Gate Unit, running Raspberry Pi OS (Debian-based Linux).

ESP32 DevKit V1: A low-power microcontroller for the Bus Unit.

RC522 RFID Reader: Operating at 13.56 MHz for reading student ID cards.

SIM800L GSM Module: Providing GPRS data connectivity and SMS capabilities.

### Software Environment

Python 3.7+: The primary language for the Raspberry Pi, chosen for its strong library support for hardware I/O and JSON handling.

Arduino IDE (C++): Used to program the ESP32 firmware.

Google Apps Script (JavaScript): Server-less backend code running on Google's infrastructure.

## 5.3 Tools and Technologies

The following technologies were instrumental in the development process:

| Category | Tool/Technology | Strict Usage |
| --- | --- | --- |
| Language | Python | Utilized for the Gate Unit logic due to its simplicity in handling threading and file I/O. |
| Language | C++ / Arduino | Utilized for the ESP32 due to its low-level hardware control and efficiency. |
| Library | TinyGSM | Critical for handling AT commands for the SIM800L module, abstracting the complexity of cellular connectivity. |
| Library | gspread | A Python API for Google Sheets, allowing the Raspberry Pi to read/write data easily. |
| Platform | Google Sheets | Chosen as a "Database-as-a-Service" for its zero-cost hosting and easy accessibility for school admins. |

## 5.4 Deployment Architecture

The deployment follows a Hybrid Edge-Cloud Architecture.

Edge Computing (Gate Unit): Critical processing happens locally on the Raspberry Pi. This ensures that students can still be scanned and parents notified via SMS even if the school internet goes down.

Cloud Computing (Bus Unit): The Bus unit relies on the cloud for validation. It connects via the cellular network (Dialog GPRS) to the Google Script API.

[Reference Diagram: Edge devices (Pi/ESP32) interacting with GSM modules and Google Cloud Platform]

## 5.5 Major Code Segments

### 5.5.1 Cloud Synchronization (Raspberry Pi)

This Python segment demonstrates the "Background Sync" feature. It ensures the local database is always up-to-date with the Google Sheet without blocking the main scanning loop.

def sync_students_from_sheet():
    global STUDENTS_CACHE
    try:
        # Auth with Google Sheets
        scope = ['https://spreadsheets.google.com/feeds', ...]
        creds = ServiceAccountCredentials.from_json_keyfile_name(CREDENTIALS_FILE, scope)
        client = gspread.authorize(creds)
        
        # Fetch all records
        sheet = client.open(SHEET_NAME).worksheet("Students")
        records = sheet.get_all_records()
        
        # Update in-memory cache and save to local JSON backup
        STUDENTS_CACHE = { str(r['UID']): r for r in records }
        with open(STUDENTS_FILE, 'w') as f:
            json.dump(STUDENTS_CACHE, f)
            
    except Exception as e:
        print(f"[CLOUD] Sync Failed: {e}")

### 5.5.2 GPRS Data Transmission (ESP32)

This C++ segment shows how the ESP32 switches from a standard function call to a GPRS network request. It utilizes the TinyGSM library to establish a cellular data connection and query the API.

// From: SmartAttendanceESP32_GPRS.ino
void sendDataToScript(String uid, String type) {
  // Construct the secure API URL
  String path = SCRIPT_PATH + "?action=handleScan" + "&uid=" + uid + "&type=" + type;

  // Initiate Request via SIM800L Modem
  http.beginRequest();
  http.get(path);
  http.endRequest();

  // Parse Response
  String responseBody = http.responseBody();
  
  if (responseBody.indexOf("\"allowed\"") > 0) {
      triggerSuccess(); // Green LED
      // Extract parent phone and send SMS...
  } else {
      triggerFail(); // Red LED
  }
}

### 5.5.3 Cloud API Logic (Google Apps Script)

This JavaScript function resides in the cloud. It acts as the central validator, receiving the UID from the bus, finding the corresponding student, and returning the parent's phone number.

// From: google_script_api.js
function handleRequest(e) {
    if (e.parameter.action == "handleScan") {
        var uid = e.parameter.uid;
        
        // Search for student in the "Students" sheet
        var student = findStudent(ss, uid);

        if (student) {
            // Log attendance to "Attendance" sheet
            ss.getSheetByName("Attendance").appendRow([new Date(), uid, student.name, ...]);
            
            // Return 'allowed' status and Phone Number to the device
            return response({
                status: "allowed",
                phone: student.phone
            });
        }
    }
}